Protocol 000: Experiment Planning

HEartLab, 2025.

# Experiment information

Date:

Time:

Experiment nº:

People:

# Pre-experiment Conference

- Check Day before sheet (Must be completed)

- Solutions preparation (check the Protocol 001)

  - Sucrose Solution:		6 L (Since the beginning)

  - Cardioplegia: 			500 ml

  - Krebs-Henseleit Solution:	10L(Langendorff  )

  - Check hardware

  - Optical cameras’ position.

  - MEAs in the right location.

  - The stimulator checking.

  - Physical space in the computer

- Check the software and recording systems

  - Check the number of electrodes in Open Ephys

  - Check if the StreamPix9 recording system is working properly

  - Check the TTL signal and camera trigger

  - Check the LabVIEW program for 3D reconstruction

  - Check pacing stimulator and software

- Fill and position the TANK next to the setup

- Check the temperature of the solutions and start recirculating

  - Langendorf Water Bath 39.5 ºC

  - Tank Water Bath - 43.5 ºC important: let this temperature set, avoiding precipitation

  - Check one by one each electrode and try to measure the solution impedance

- Start oxygenation of the solutions (At least 10 min before heart connection)

- Remove bubbles from the system

- Start OBS recording

# Experiment Procedures

## Heart stabilisation

- Euthanasia and heart extraction Protocol: Euthanasia

- Put the extracted heart inside one beaker with the solution previously

- Cannulate the heart with the special reference cannula and connect to the Langendorff system. Important: Position not too deep in the aorta

- Cleaning the heart: Remove the lung tissue surrounding the pulmonary veins. Separate the pulmonary artery from the aorta and insert a thin tube to prevent blockage.

- Wait at least 10 minutes for stabilisation

- Balloon insertion in the atria.

- Oriented the heart as follow, LA=cam A-MEA3; RA=cam B-MEA1; V=cam C-MEA2;

## Recording reference for Sinus

- Starts OBS recording

- Recording electrical signal only - Without Dye or Blebbistatin

## Dye Administration and Heart Movement Blockage

- Dye Infusion - Protocol 005: Dye Infusion

- Remember to let the air out of the ballons during infusion and to let the air in after infusion

- Recording electrical SN.

- Blebbistatin Infusion - Protocol 006: Movement Suppression

- Wait for 15 minutes for stabilisation

- Record optical and electrical SN

- Test pacing before submerging into the tank

## Tank Positioning

- Put the tank in the middle of the setup

- Put the heart in the tank full of Sucrose solution and preheated to 37 degrees.

- Orient the heart as follow, LA=cam A-MEA3; RA=cam B-MEA1; V=cam C-MEA2;

## Cardiac Baseline Mapping

- Record optical and electrical SN

## Pacing

- Position the pacing electrode over the LA - register the place.

- Record Pacing 350, 300, 250ms. (Two recording of 10 seconds each)

## Baseline Effective Refractory Period (ERP)

- Program the stimulation software with:

  - S1= 250ms (8 pulses)

  - S2= starting in 220ms (1 pulses) and decreasing by 10ms, decrease by 2ms close to the ERP

  - Important: Identify the longest S1S2 coupling interval with loss of AV-nodal conduction. This interval corresponds to the AVNERP (AV node effective refractory period)

- Record the ERP and one before that

## Drugs protocol

- Dilute 1ml of a 1mM Carbachol solution (1 Eppendorf) in 999 ml of nutrient solution.

- Dilute 25 ml of Concentrated Blebbistatin (68 uM at 30°C) to that 1L solution.

  - Following the same protocol for “Motion Suppression”.

- Add the eluted 1uM carbachol solution into the reservoir with a low level of solution.

- Wait for up to 30 seconds and repeat Baseline Effective Refractory Period (ERP)

- Before the electrical stimulation, we need to see a consistent decrease in heart rate rhythm.

## Arrhythmia Induction

We start with the atria if we do not have results we try to induce ventricle arrhythmia.

- Burst Protocol

  - Duration: 2s each burst

  - Pulse Width: 2ms

  - Cycle Length: 20ms (50Hz)

  - Number of Cycles: 8

  - Interval between cycles: 10s

  - Intensity: Maximum possible

- Wait ten minutes between each protocol

- Repeat a maximum of 5 times the protocol

## 3D Generation

- Remove the CO2 supply and disconnect the Langendorff system

- Identify the camera to be used in 3D reconstruction in the sheet with the figure

- Make the heart 3D mapping of the heart in the tank

- Make the 3D calibration map with the tank

- Remove the tank from the setup

- Make the 3D calibration map without the tank

- Make the 3D mapping of the heart without the tank