Protocol 001: Solutions

HEartLab, 2026
Guide to reagents and concentrations for preparing solutions.

Solution preparation

● In distilled water, mix the quantities listed in the table according to the required final volume.
Important: CaCl₂ must be dissolved separately to avoid precipitation.

● Filter the solution using filter paper (specify the type or pore size if needed).

● With the Langendorff Setup, put the complete solution in the reservoir and flow carbogen through it for at least 10 minutes, let it flow at a rate of 30mL/min, and after 3 minutes, collect 30mL of solution in a 50mL beaker. Measure the pH immediately if the pH is not around 7.4 or less. Correct the solution with 1mL of 0.1M of HCl for every 1L of solution and repeat the process.

Modified Krebs–Henseleit solution (General purpose)

|  | Con. (mMol/L) | Molar mass (g/Mol) | g/0.5 L | g/1 L | g/2 L | g/4 L | g/10 L | g/12L |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NaCl | 115 | 58.44 | 3.3603 | 6.7206 | 13.4412 | 26.8824 | 67.2060 | 80.6472 |
| KCl | 4.6 | 74.55 | 0.1715 | 0.3429 | 0.6859 | 1.3717 | 3.4293 | 4.1152 |
| NaHCO3 | 25 | 84.01 | 1.0501 | 2.1003 | 4.2005 | 8.4010 | 21.0025 | 25.2030 |
| CaCl2 . 2H2O | 1.2 | 147.02 | 0.0882 | 0.1764 | 0.3528 | 0.7057 | 1.7642 | 2.1171 |
| KH2PO4 | 1.2 | 136.09 | 0.0817 | 0.1633 | 0.3266 | 0.6532 | 1.6331 | 1.9597 |
| MgSO4 . 7H2O | 1.2 | 246.48 | 0.1479 | 0.2958 | 0.5916 | 1.1831 | 2.9578 | 3.5493 |
| NaPyruvate | 1 | 110 | 0.0550 | 0.1100 | 0.2200 | 0.4400 | 1.1000 | 1.3200 |
| Glucose | 11 | 180.16 | 0.9909 | 1.9818 | 3.9635 | 7.9270 | 19.8176 | 23.7811 |

Cardioplegia

| Reagents | Molar Mass (g/mol) | Molarity (M) | g/L | g/ 0.5L |
| --- | --- | --- | --- | --- |
| NaCl | 58.44 | 0.11 | 6.43 | 3.215 |
| KCl | 74.55 | 0.016 | 1.19 | 0.595 |
| NaHCO3 | 84 | 0.01 | 0.84 | 0.42 |
| MgCl2.6H2O | 203.33 | 0.16 | 32.5 | 16.25 |
| CaCl2.2H2O | 147.02 | 0.0012 | 0.176 | 0.088 |

Sucrose Electrolytic Solution

| Reagents | Molar Mass (g/mol) | Molarity (M) | g/L | g/4L | g/2L |
| --- | --- | --- | --- | --- | --- |
| NaCl | 58.44 | 0.023 | 1.3441 | 5.3764 | 2.6882 |
| Sucrose (Sacarose) | 342.3 | 0.26 | 88.998 | 355.992 | 177.996 |
| Target Conductivity |  | 500Ωcm = 2mS/cm |  |  |  |

Special Reagents

Carbachol Stock (1mM)

- Molar Mass: 182.7 g/mol.

- For 1 mM: 0.183 mg/ml.

- Prepare 20 ml and distribute into 20 Eppendorf tubes with 1 ml each.

- Dissolve 3.66 mg in 20 ml of water.

- The working solution is 1 µM.

- Dilute 1 ml (1 Eppendorf) in 999 ml of nutrient solution.

Dye (0.25 mg per Eppendorf)

- Follow Dye Protocol

Blebbistatin (1 mg per Eppendorf)

- 1 Eppendorf contains:

- 1 mg of (-)-Blebbistatin or (+-)-Blebbistatin

- 200 µl of DMSO.