Protocol 005: Dye Infusion

HEartLab, 2025
Infusion of the Dye (Di-4-ANBDQPQ) for Langendorff isolated rabbit heart

# Procedure

- Once the heart is appropriately accommodated to the perfusion for about 15 minutes, the dye can be added.

- Dissolve one Eppendorf tube containing 0.25 mg of the dye into the 50 ml of Nutrient Solution heated at 38°C in a Becker. Regarding heating the nutrient solution, it will work equally well if you heat it to 30°C or keep it at room temperature.

- Turn off the peristaltic pump that fills the upper flask reservoir.

- Then, the dye, which has been dissolved, is added to the upper flask reservoir from the Langendorff system, which is filled up to approximately 100 mL or more.

- Use a beaker to collect the perfusate and reuse it for infusion.

  - Reuse until the solution is clear, generally 1 or 2 times.

  - Remember to filter the solution in the vacuum pump before reperfusing it.

- The perfusion can then continue until the upper flask reservoir is empty.

- Turn back the peristaltic pump and continue to perfuse the regular perfusion solution.

# Observations

- To ensure optimal distribution of the dye throughout the heart, it is recommended to maintain a controlled flow rate. It is advised not to exceed a flow rate of 20 ml/min, allowing the dye to slowly reach all regions of the heart. In that manner, it will take 5 to 10 minutes until the 250 ml empties.

- For each rabbit heart, an Eppendorf tube containing 0.25mg of the dye is typically used, which is more than enough for the procedure. In a recent experiment, we successfully utilized only 0.75mg of the dye for an entire human heart, whereas the usual dosage is 2mg.