Protocol 006: Heart Movement Blockage

HEartLab, 2025
Blockage of the movement of the heart due to its contractility

# Procedure

- Each Eppendorf tube contains 1mg of (+)-Blebbistatin (MM = 305.38 g/mol). Blebbistatin concentration in the nutrient solution is recommended at least 1,7 uM (approximately 1mg of Blebbistatin for 2 litres of the nutrient solution).

  - Obs1: In some cases, 1.7uM may not be sufficient, and it may be necessary to increase the concentration.

  - Obs2: Avoid dissolving Blebbistatin at room temperature.

  - Obs3: The temperature of the tubes containing Blebbistatin, and the small nutrient solution volume should be close to each other.

  - Obs4: The best is to have the Eppendorf with (+)-Blebbistatin floating inside a beaker glass filled with water, heated by a heating plate, and in the same plate the 50 to 100 ml of Tyrode volume is also heated, in the same temperature.

- To prepare the Blebbistatin, pour the content of the tube into a 30°C heated 50 ml of nutrient solution. It will generate a 66 μM Solution.

- After dissolving check that the colour of the solution is transparent.

- Add the Blebbistatin to the perfusate as follows:

  - For initial motion suppression, add 25 ml of concentrated solution (66 uM) containing dissolved Blebbistatin to the upper flask reservoir, containing 25-75 (50) ml of nutrient solution, in the Langendorff setup.

    - This will result in an initial concentration of approximately 22 μM. If the flask is filled with 50 ml of nutrient solution.

  - Mix the remaining solution containing 25 ml of Concentrated Blebbistatin (66 uM) to the main nutrient solution 1L reservoir. This way the final Blebbistatin concentration is 1.7 uM.

# Observations

- A higher temperature of around 40°C works better but can be more challenging and result in precipitates from the nutrient solution. If the temperature is too low, Blebbistatin may precipitate.

- Please check the heart, and also throughout the cameras, if there is any mechanical movement. If everything goes well, the high concentration will cease heart concentration as soon as the Blebbistatin reaches the heart, and the low concentration nutrient solution will maintain the steady state. Usually, high concentration works well, and the effect is fast. During the experiment, if the heart starts to contract it is an indicator that a higher than 1.7 uM Blebbistatin concentration is needed and usually another high concentration infusion of Blebbistatin is needed.

- PS: If you use (±)-Blebbistatin, it will be necessary to double the concentration.